Cytec PC-16MX, 16 Channel 2 Pole Reed Relay Multiplexer

-------------------------------------------------------

Included on the distribution disk are files to operate the PC-16MX

module from the LabWindows environment.  The programs provide

initialization, channel selection and status return functions.

Files:

PC_16MX.C      C Source Code.

PC_16MX.H      C Function Prototypes.

PC_16MX.BAS    Basic Source Code.

PC_16MX.INC    Basic Include File.

PC_16MX.FP     Panel Controls.

PC_16MX.LBW    PC_16MX Instrument Program.

PC_16MX.LWI    PC_16MX Instrument Include.

Functions:

Initialize -   Sets the hardware port address and opens all

channels.

Select     -   Selects 1 of 16 channels, may be used to open

all channels.

Status     -   Returns the last selected channel or all open

status.

--------------------------------------------------------------

Function:	Initialize

Initialize Hardware Port, Clear Channels.

Syntax C:	int mx16_initialize(port)

int port;

Syntax Basic:	status% = mx16.initialize%(port%)

Inputs:		Port : Integer

Hardware Port Address.

Returns:	Status : Integer

0 - Initialize OK.

NZ - Errors.

--------------------------------------------------------------

Function:	Select

Select a Channel to Close, all others opened.

Syntax C:	int mx16_select(channel)

int channel;

Syntax Basic:	Status% = mtx16.select%(channel%)

Inputs:		Channel : Integer

Channel to close, range 0 through 15.

Any other input will open all channels.

Returns:	Status : Integer

0 through 15 Closed Channel.

-1 All open.

--------------------------------------------------------------

Function:	Status

Returns channel status.

Syntax C:	int mx16_status();

Syntax Basic:	Status% = mtx.status%

Returns:	Status : Integer

0 through 15 Channel Closed.

-1 All open.