Cytec PC24/D

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Cytec PC24/D.

It contains functions for opening and configuring the output driver

channels.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

The port address as set by the DIP switch on the module must be known.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Cytec PC24/D

instrument driver. The functions are listed in alphabetical order with

a description of the function, C and QuickBASIC syntax of the function,

a description of each parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Cytec PC24/D                         Function Name:

Initialize                        d24_initialize

Lines                             d24_line

Read                              d24_read

Write                             d24_write

________________________________________________________________________________

Instrument         : Cytec PC24/D

Cytec CP24/D

This module allows control of the PC24/D Relay

Driver Module.

Initialize - Call before any operations to set the

base I/O address.

Lines - Modifies a single line on the specified

port.

Read - Reads the Byte value from the specified

port.

Write - Sends a Byte to the specified port.

________________________________________________________________________________

Following functions are in alphabetical order.

________________________________________________________________________________

Function Name      : d24_initialize

C Syntax           : int d24_initialize(Base_Address);

int Base_Address;

QuickBASIC Syntax  : Status% = d24.initialize(Base.Address%)

Purpose            : Initialize

Set the Base Address for subsequent I/O

operations.

Returns all ports added after initializing -

this value should be 0.

Parameter List     : Base Address

Integer - Base Address

The physical I/O port address as set by the

DIP switch on the module.

Return Value       : Status Return

0 - Initialized OK.

NZ - Problems.

________________________________________________________________________________

Function Name      : d24_line

C Syntax           : int d24_line(Port, Line, OnOff);

int Port;

int Line;

int OnOff;

QuickBASIC Syntax  : Status% = d24.line(Port%, Line%, OnOff%)

Purpose            : Lines

Allows modification of a single line on the

specified port.

Line - Line # to modify, 0 - 7.

Port - Port to modify line on.

On/Off - Turn the specified line On or Off.

Returns - Port Byte value input.

Parameter List     : Port

0 - 0 (Default)

1 - 1

2 - 2

Port to be modified

Line

0 - 0 (Default)

1 - 1

2 - 2

3 - 3

4 - 4

5 - 5

6 - 6

7 - 7

Line to be modified.

On/Off

1 - ON (Default)

0 - OFF

Turn line # On or Off.

Return Value       : Status

Byte read from port after line modification.

________________________________________________________________________________

Function Name      : d24_read

C Syntax           : int d24_read(Port);

int Port;

QuickBASIC Syntax  : Status% = d24.read(Port%)

Purpose            : Read

Reads and Returns the Byte value from the

specified port.

Port to read from.

Return Value       : Status

Byte read from port.

________________________________________________________________________________

Function Name      : d24_write

C Syntax           : int d24_write(Port, Pattern);

int Port;

int Pattern;

QuickBASIC Syntax  : Status% = d24.write(Port%, Pattern%)

Purpose            : Port Write

Sends the Byte pattern to the specified port.

Port - 0, 1 or 2.

Pattern - 0 to 255.

Returns - Byte input from specfied port.

Parameter List     : Port

0 - 0 (Default)

1 - 1

2 - 2

Port

Port to send the pattern out to.

Pattern

Pattern

Byte to send to the specified port.

Return Value       : Status

Byte read after the write from the specified port.